Here is the step-by-step connection guide for your ESP8266 (Wi-Fi Mode) project.

Since we are using the Wi-Fi method (because the ESP8266 does not have Bluetooth), we will connect the LED to a digital pin on the board.

The Goal

We are creating a circuit where electricity flows from Pin D1 ->Through the LED ->Through the Resistor ->To GND (Ground).

1. Identify Your Components

Before connecting, make sure you know which leg is which:

- LED: Look at the legs. The Longer Leg is Positive (+). The Shorter Leg is Negative (-).

- Resistor: It has no polarity. It works both ways.

- ESP8266 (NodeMCU): Look for the pin labeled D1 (sometimes labeled GPIO5) and the pin labeled GND.

2. Step-by-Step Connections

You can build this on a breadboard or by twisting wires together.

Step A: Connect the Positive Side

- Take a Jumper Wire.

- Plug one end into Pin D1 on your ESP8266.

- Connect the other end to the Long Leg (+) of your LED.

Step B: Connect the Negative Side

- Take the Short Leg (-) of the LED.

- Connect it to one end of the 1 kΩ Resistor. (If using a breadboard, put them in the same row).

Step C: Connect to Ground

- Take the other empty end of the Resistor.

- Connect it to the GND pin on your ESP8266.

3. Connection Summary Table

| Component | Pin / Leg | Connects To |
| --- | --- | --- |
| ESP8266 | Pin D1 | LED Long Leg (+) |
| LED | Short Leg (-) | Resistor (One End) |
| Resistor | Other End | ESP8266 GND |

4. Detailed Breadboard Guide (If you are using one)

If you are using a white breadboard with holes, follow these exact coordinates to avoid short circuits:

- Place the ESP8266: Push the ESP8266 into the breadboard so it straddles the middle gap.

- Place the LED:

  - Put the Long Leg into a free row (let's call it Row 10).

  - Put the Short Leg into a different free row (Row 11).

- Wire the Signal:

  - Run a wire from ESP8266 Pin D1 to Row 10 (where the Long Leg is).

- Place the Resistor:

  - Put one leg of the resistor into Row 11 (connecting it to the LED's Short Leg).

  - Put the other leg of the resistor into a new row (Row 15) or directly to the blue rail (-) on the side.

- Wire the Ground:

  - Run a wire from ESP8266 GND to the same place as the resistor's second leg (Row 15 or the blue rail).

Upload the code to your ESP8266. It creates a web page to control the LED.

🔐 Replace:

- "YOUR_WIFI_SSID" → with your Wi-Fi name.

- "YOUR_WIFI_PASSWORD" → with your Wi-Fi password.

📱 STEP 5: Use from Phone or Computer

- Open Serial Monitor (set to 115200 baud) in Arduino IDE.

- After connecting, it will display the ESP8266’s IP address (e.g., 192.168.1.100).

- On your phone or PC (connected to the same Wi-Fi), open a browser and enter that IP.

- You’ll see buttons to turn LED ON/OFF.

Note:  It works only for 2.4 Hz connection